Basic Operation

The Atlas fencing machine is a three-weapon device compliant with FIE regulations.

Front View

At the top of the machine, there are two 8 × 8 LED panels that display the relevant information for each fencer. When no hit has been registered, each panel shows the score, cards, including both standard and non-combativity cards, and the non-combativity time.

When a valid or non-valid hit is registered, this information disappears. The panel turns red or green for a valid hit, or displays a white cross for a non-valid hit.

Standard cards are shown as two yellow or red dots in the lower part of the panel.

Non-combativity cards are shown as a single yellow or red dot in the lower part of the panel.

Black cards are shown as a blue cross covering the entire screen.

Non-combativity time is displayed as a bar on the inner side of the panels. Each dot represents 10 seconds: the first 50 seconds are shown with blue dots, the last 10 seconds with an orange dot, and any additional time beyond that with a red dot.

Unless specifically stated all the operations described can be performed both with the dedicate remote or the Android application.

Reprogramming the Atlas Machine

Firmware updates should be carried out through the machine’s OTA Portal. To install new software on the machine, the user must first activate OTA mode using the remote control.

Once OTA mode has been activated, the computer must be connected via Wi-Fi. The machine creates a Wi-Fi hotspot named Piste_XXX, where XXX corresponds to the piste number. The default password is 01041967.

After connecting the computer to the machine, enter the following address in your preferred web browser:

http://192.168.4.1/update

Select the OTA firmware update file and start the flashing process. Wait until the upload reaches 100%. Then reboot the machine and confirm that the new firmware version has been installed successfully.

Auto Referee Mode for the Atlas Machine

- Select the weapon you wish to fence with.

- Both fencers must perform a long double hit to activate the mode.

- The red and green lights will illuminate from the center outward, indicating that the mode has been activated.

- After 8 seconds, the white light will flash and the timer will start.

- When a touch is scored, the point is awarded automatically, except in the case of a double hit in foil or sabre. In that case, the fencer who is to receive the point must make an additional hit to confirm and register the score. The timer will resume 8 seconds later.

- A quick double hit, whether valid or invalid, will reduce the score if necessary.

- P-cards are assigned automatically.

- Periods, breaks, and priority are also managed automatically.

- Once the match has ended, no further points can be awarded. To restart a match, both fencers must perform a long double hit.

- The remote control retains all functions in this mode.

Modo Árbitro Automático para la Máquina Atlas

- Seleccione el arma con la que desea tirar.

- Ambos tiradores deben realizar un doble tocado largo para activar el modo.

- Las luces roja y verde se encenderán desde el centro hacia afuera, indicando que el modo ha sido activado.

- Tras 8 segundos, la luz blanca parpadeará y el cronómetro se pondrá en marcha.

- Cuando se produce un tocado, el punto se concede automáticamente, excepto en caso de doble tocado en florete o sable. En ese caso, el tirador que deba recibir el punto deberá realizar un tocado adicional para confirmar y registrar la puntuación. El cronómetro se reanudará 8 segundos después.

- Un doble tocado rápido, válido o no válido, reducirá la puntuación si es necesario.

- Las tarjetas P se asignan automáticamente.

- Los periodos, las pausas y la prioridad también se gestionan automáticamente.

- Una vez finalizado el combate, no podrán concederse más puntos. Para reiniciar un combate, ambos tiradores deberán realizar un doble tocado largo.

- El mando a distancia conserva todas sus funciones en este modo.

Mode Arbitre Automatique pour la Machine Atlas

- Sélectionnez l’arme avec laquelle vous souhaitez tirer.

- Les deux tireurs doivent effectuer une double touche longue pour activer le mode.

- Les voyants rouge et vert s’allument du centre vers l’extérieur, indiquant que le mode a été activé.

- Après 8 secondes, le voyant blanc clignote et le chronomètre démarre.

- Lorsqu’une touche est portée, le point est attribué automatiquement, sauf en cas de double touche au fleuret ou au sabre. Dans ce cas, le tireur qui doit recevoir le point doit effectuer une touche supplémentaire pour confirmer et enregistrer le score. Le chronomètre reprend 8 secondes plus tard.

- Une double touche rapide, valable ou non valable, réduit le score si nécessaire.

- Les cartons P sont attribués automatiquement.

- Les périodes, les pauses et la priorité sont également gérées automatiquement.

- Une fois le match terminé, aucun point supplémentaire ne peut être attribué. Pour redémarrer un match, les deux tireurs doivent effectuer une double touche longue.

- La télécommande conserve toutes ses fonctions dans ce mode.

Automatische Scheidsrechtermode voor de Atlas-machine

- Selecteer het wapen waarmee je wilt schermen.

- Beide schermers moeten een lange dubbele treffer uitvoeren om de modus te activeren.

- De rode en groene lampjes lichten op van het midden naar buiten, wat aangeeft dat de modus is geactiveerd.

- Na 8 seconden knippert het witte lampje en start de timer.

- Wanneer een treffer wordt gemaakt, wordt het punt automatisch toegekend, behalve bij een dubbele treffer in floret of sabel. In dat geval moet de schermer die het punt hoort te krijgen een extra treffer maken om de score te bevestigen en te registreren. De timer wordt 8 seconden later hervat.

- Een snelle dubbele treffer, geldig of ongeldig, verlaagt indien nodig de score.

- P-kaarten worden automatisch toegekend.

- Periodes, pauzes en prioriteit worden eveneens automatisch beheerd.

- Zodra de wedstrijd is afgelopen, kunnen er geen verdere punten meer worden toegekend. Om een wedstrijd opnieuw te starten, moeten beide schermers een lange dubbele treffer uitvoeren.

- De afstandsbediening behoudt in deze modus al haar functies.